CIRCUIT ANALYSIS LAB_1

Name:    Muhammad Uzair

Reg No: 2025-CE-112

Date:      16thMarch,2026

POST LAB QUESTIONS & ANSWERS:

### 1. Define Oscilloscope and write its applications.

An Oscilloscope is an electronic device used to display electrical signals in waveform form. It helps in measuring voltage, frequency, and time period of signals. Oscilloscopes are widely used in electronics laboratories and communication systems. They are also used for troubleshooting electronic circuits and observing AC/DC waveforms.

### 2. What is Peak-to-Peak Voltage?

Peak-to-peak voltage is the total voltage difference between the highest positive peak and the lowest negative peak of a waveform. It represents the complete amplitude of the signal. For a sine wave:

Vpp=2Vmax

It is commonly measured using an oscilloscope.

### 3. What is RMS Value?

RMS (Root Mean Square) value is the effective value of an AC voltage or current. It is equal to the DC value that produces the same heating effect in a resistor. RMS values are important in electrical and electronic calculations. For a sine wave:

Vrms=Vmax/1.414

### 4. Write the RMS Values of Sine, Square and Triangular Wave.

The RMS value depends on the shape of the waveform. For a sine wave:

Vrms=0.707Vmax

For a square wave:

Vrms=Vmax

For a triangular wave:

Vrms=Vmax/1.73

### 5. Explain DC Offset.

DC offset is the addition of a constant DC value to an AC signal. Due to DC offset, the waveform shifts above or below the zero reference line. It affects the average value of the signal. DC offset is important in signal processing and electronic communication systems.